Engine “galloping”

Engine “galloping” means the maximum rotation speed cannot be controlled." The galloping failure is usually caused when the injection pump control gear rod is lock up or speed regulator flyweight and slide block are seized/ So during pump calibration, it is required to check if movement of control gear level is agile, and slide resistance is in standard range. Light fuel may also lead to “galloping” accident, so light fuel shall be blown dry (or wipe dry) with compressed air and then install when cleaning engine air intake system or parts of fuel system (such as diesel filter), and light fuel (such as diesel or kerosene) is needed during maintenance.