Engine oil consumption is high

There are mainly two reasons for the high engine oil consumption:

Engine combusts oil or engine discharges oil (leakage). Normally, engine oil consumption and fuel consumption ratio is less than 0.8%; it means it is normal phenomenon that every consumption of 100 liters fuel, engine oil consumption is less than 0.8 liters. Take 1419.280/043.6×4 type standard van truck as example, vehicle constant speed(50km/h) fuel consumption is 33 liters per 100 kilometers, that means it is normal phenomenon that every running of 300 kilometers, engine consumption is about 0.8 liter, if oil consumption is more than this value, it means there’s failure on engine.

The main reasons of “oil combustion” are as follows:

1. Too much oil is added The oil on the bottom of engine can not be added excessively; on the one hand, filling it with too much oil will make oil temperature too high, on the other hand, too much oil splashed over the cylinder will make burden on scraping of oil on piston ring, and when oil in the cylinder is too much from pump, engine oil will become excessive and burnt.

2. Gap in piston and cylinder sleeve is too big. Due to big gap in piston and cylinder sleeve, oil left on cylinder wall increases, thus oil is excessive and burnt. The failure always follows with crankshaft case exhaust increasing.

3. Piston ring open faces wrong direction or sticky ring. Due to careless installation (three piston ring opens should be staggered with 120°), piston ring open staggered with small angle, thus three piston opens go another side (also called ring joint) during running. Or cylinder score caused to piston ring and ring groove completely stuck (also called sticky ring), thus scraping function of piston ring is in failure, and oil burning failure When this failure occurs, exhaust of crankshaft case will increase.

4. “Cylinder score” causes oil burned out If cylinder score accident happens, it causes not only exhaust increasing, decrease of power, cylinder knocking, but also oil burning failure.

5. Wrong installation of the second air ring caused engine oil to burn.  There are two air rings and one oil ring on piston of WD615 series diesel engine. The section of oil ring is in symmetrical completely, so there’s no difference of upper and lower, side and reverse side during installation. The first air ring is double face barrel ring; the ring face marked with “TOP” (up) shall be upward during installation. However, there’s groove on the upper ring face of first air ring, thus it will not installed incorrectly during installation. The second air ring is taper face ring, that is, inner ring face is square ring face, and outer ring face is cone type ring face However, the taper angle of outer ring face is too small, and it is hard to visually observe; so usually, repair technician considers it as square ring by mistake, and pay no attention to upper and lower ring face during installation. If lower ring face is installed upward, the cone ring scraping effect will become oil effect on upper, and it will bring oil burning failure obviously. As a result, when installation on the second air ring, the ring marked with “TOP” shall be installed upward.

“Engine oil combustion” failure can be seen from engine smoke obviously. Generally, engine emission with blue smoke is dazzling, so engine is burning oil.

There are following factors for main reason of “oil exhaust” failure

1. Engine oil leaks. Usually, the leakage position on engine occurs in front and rear crankshaft oil seal, timing intermediate gear shaft, etc. Oil seal quality problem and careless installation will cause oil leakage, and crankshaft case air vent device; likewise, if oil air separator is blocked, crankshaft oil seal will leak as well. Check oil air separator at the front end of engine frequently during usage

2. Oil leakage of supercharger. Due to long time wrong operation, supercharger is worn out and leakage occurs early, which are the main reasons of engine oil consumption. In the more serious case, engine oil can be spilled when dismantling pipes between supercharger and intercooler.

3. Air compressor oil exhaust excessively big gap of air compressor cylinder sleeve piston may cause oil exhaust as well. Oil discharged from compressor running through pipe line to oil water separator of gas pipe, then discharged to air by automatic dirty emission valve.

4. Oil with too low viscosity also causes increasing oil consumption.