Oil level in engine oil pan is increasing

In real operation, sometimes oil does not decrease but increases obviously in oil pan. This is mainly because water, diesel or hydraulic oil mixes with engine oil in the oil pan.

The main reasons why there’s water in oil include:

1. Cylinder pad is worn out to make oil channel connect with water channel. So coolant leaks out into the oil pan and oil enters into coolant.

2. There’s hole or crack in water channels of cylinder block and cover, leading to leakage of coolant into the oil pan.

3. Plug of water channels in the cylinder cover is loose and rusted, leading to leakage of coolant into the oil pan.

4. Insulation bush of injector and cylinder cover is not sealed well, leading to water leakage.

5. Core of oil radiator is loose welding or worn out, and oil leaks into coolant during running of engine, but coolant can leak into oil while not working. Coolant mixed into oil leads to emulsify of oil, and coolant can be discharged from oil release bolt in more serious case.

Oil will be diluted when it contains diesel, and also has smell of diesel.  The main reasons are:

1. Injector (some cylinders) needle valve is seized, injector nozzle is burned out, injection pressure is too low and oil spray cannot be atomized, and unburned big fuel particles pass cylinder wall and flow into oil pan. The failures will not only increase oil, and when it is serious, diesel might flush cylinder wall, and if lubrication is bad, it will also lead to scuffing phenomenon.

2. Injector pump plug leaks seriously, and diesel is mixed with engine oil through the injector pump oil pan, and then flowing into the oil pan through the oil return pipe. If the oil supply pump piston is worn out seriously, it will also cause similar failure.

Diesel leaks into the oil pan and dilutes the engine oil to worsen oil intensity and lower viscosity, leading to bearing burning incident in more serious case.

The only reason that hydraulic oil and oil are mixed is that hydraulic pump rotation axle oil seal has oil leakage, thus hydraulic oil enter into timing chamber, and flowing to the oil pan. The mixture of hydraulic oil and oil has no obvious characteristic. As to whether mixture exists or not, it can only be judged when hydraulic oil in hydraulic tank reduces and no obvious oil leakage is found outside.