The main reasons that engine bearing bushing is burnt

Common faults for shaft bushing of engine and linkage bushing are  “bearing bushing burning” “bearing score” and” damage bearing”.

“Bearing score” occurs mainly due to dirty oil which contains micro mechanical purities mixed in engine oil running to space between the bearing and the bushing. Solid purities may make damage on alloy in the bushing, and even pulling and damaging the crankshaft journal bottle in more serious case.

“Damage bearing” occurs due to big gap between bearing journal and the bearing bushing, leading to poor oil quality and inadequate intensity, so oil slick does not exist on the impact of shaft and bushing, and alloy in the bushing cracking and even droping in more serious case.

“Bearing burning” is a comprehensive failure of bearing bushing. It is mainly because of poor lubrication. In more serious case it happens that the bearing bushing and journal are burnt, leading to rolling of bushing. Bearing buring occurs as well because gap between the main bushing and the linkage bearing gap is too much, or because partial lack of lurbricating oil occurs due to oil leakage and low pressure and insufficient oil supply, or because mechanical purities or oil stains block oil channels, or oil pump filter falls apart, or oil pan is in lack of oil, etc.  “Bearing score” and “damage bearing” will cause burning bearing incident

Usually, pull and damage the bearing bushing, and then burn bearing bushing due to low oil pressure and lack of oil. So, it is required to pay attention to oil pressure and oil level warning lamp at all time during usage. When oil pressure drops suddenly or warning lamp illuminates, it is required to immediately stop the engine and check it.

If mating between the main bearing bushing or linkage bearing and seat hole is poor, it often causes first rolling and then burning of bearing bushing.  That is, oil channel is blocked by bearing bushing rolling in the bushing seat, and then bearing is burned for lack of oil. Camshaft bush and cylinder body seat hole is too small, and will block the oil patch by bush.

It is important to use oil with correct brand and model. WD615 series diesel engine is high intensified diesel engine, and should use AP1CD grade oil (300 hp above EuroⅡand EuroⅢ engine should use AP1CF grade oil), which should be guaranteed. If other low grade oils are used, bearing burning incident will occur soon inevitably. Oil viscosity is also critical. Engine is not sensitive to oil viscosity, and too low oil viscosity will result in lubrication failure.  So it is required to follow requirement of user manual, and choose oil viscosity in reasonable manner according to surrounding temperature. SAE15W/40 label viscosity oil can be used in whole year except in cold area.

Some users make repair by grinding and cutting of crankshaft when it is burned, but they do not know tuff riding heat treatment on bottle, and then install directly after fine grinding, as a result, it is burned or worn out seriously in short time. This is a valuable lesson.

The failures of engine are various, and here it is just an analysis according common failures in reality. When failure happens, judge failure position first according to practice, and then dismantle and find cause of failure. Hurried disassembly without analysis will make easy things to be difficult.