1.2 Rail pressure cannot set up (starter can work normally, but do not start)

Common rail system has high requirement for fuel circuit, low pressure fuel circuit (fuel tank, coarse filter, fine filter, oil return), high pressure circuit (high pressure fuel pump, common rail, high pressure oil pipe, fuel injector) need sealing, any segment appeared problem, rail pressure can not normally set up, Hint: vehicle manufacturer tend to be highly concerned with complete fuel oil circuit

◆Check if fuel tank oil level is too low

◆Check if work of handle pump is normal

◆Check if there is air in low pressure pipelines, and exhaust air (sometimes, leakage of low pressure pipeline is not apparent that needs careful check.)

Air exhaust method, mainly exhaust air in the coarse filter, loosed drain bolt in the coarse filter, press down hand pump by hand until oil comes out continuously in the drain bolt.

Hand

pump

Ventage bolt

Oil return hollow bolt

◆After air has completely emptied in the low pressure pipeline, diesel engine can not start. Check if there is air in high pressure pipelines have air, at the same time, need to exhaust air in high pressure pipelines,

Air exhaust method: Loosed high pressure pipe of one cylinder. Drive diesel engine by starter until high pressure pipe until oil comes out continuously.(It is not suggested to frequently remove high pressure oil pipe connector)

◆Check if there is leakage in high pressure pipeline.

◆Check if oil pipeline is clear or diesel filter is blocked. It is suggested to replace diesel filter element

Check method: Loosed outlet bolt of fine filter, drive diesel engine by start, and check if diesel spurted or flow out. If little diesel spurted only, then it can be confirmed that filter element was blocked

◆Check if initial voltage value of rail pressure sensor achieve about 500mV, or setting rail pressure is 300-500bar(diagnosis by KTSscan tool). If abnormal, firstly check if connectors are firm. If no inspecting equipment available, pull out rail pressure sensor, and try to start again;

◆ Check if flow metering valve is normal, pull out connector and try to start again.