Engine starting is abnormal

Diesel engine series: WP12.336N

Failure time: 2008.04.16

Failure description: At the first start, the engine cannot be started. Then start the engine again and it is successful.

Failure reason analysis:

The engine starts with synchronization signal problem or harness problem, it bursts for 3 seconds, with “Tutu” cylinder burst sound, and the exhaust pipe discharges black smoke and rotating speed is 400-500r/min, and the engine stops automatically, then the engine is started, and it will start successfully with starting time less than 1 second. If power-off time is too long, same automatic stop happens again at the first starting. When started, if flash code lamp is on with 5-4-1 flash code (error path 3.6),  it is a synchronization signal problem. Check camshaft phase sensor and crankshaft rotating speed senor connector, and check sensor harness, and they are all normal; then collect and analyze data, and it is confirmed that the camshaft sensor malfunctions; then change high pressure fuel pump, and problems are solved, the vehicle is started successfully.

The reason is that cylinder identification gear inside the Bosch high pressure fuel pump is not machined in a correct manner.

Weichai GB 3 series diesel engine is equipped with the Bosch generation II high pressure common rail system. High pressure common rail system has high requirement of fuel pipe (including low pressure fuel pipe and high pressure fuel pipe), and many problems are aroused by fuel pipe in servicing. Vehicle harness is also another main source of failure. These two causes should be taken into consideration at first.