Failure diagnosis and remedies

1. When diesel engine malfunctions, check suspicious components one by one from easy to difficult judgment.

2. If causes are not identified, do not replace any component.

3. If causes are not identified, do not clear flash code, and please record flash code!

Troubleshooting steps:

1. Read flash code according to flash code lamp, and confirm failure point; if it is impossible to confirm it, turn to next step

Two methods can be used to read flash code: :

Read it by diagnostic instrument;

Or read it by flash code lamp of diesel engine

Reading of flash code:

When diagnostic switch shows a pulse signal, flash code lamp gives one flash code; when diagnostic switch is connected; flash code lamp is constantly alight!  ECU error internal memory can only store 10 errors in maximum; and off for 0.5s1s.

Each flash code consists of three numbers, and each number has one second interval. E.g.:321

On

Off

0.5s

1s

0.5s

1s

Clear flash code manually

Press (connect) diagnostic switch before connection of T15, and then turn on the key switch (T15 connection), maintain diagnostic switch for 4-8 seconds and then release.

If certain flash code cannot be deleted, it is current failure.

Hardware failure shall be eliminated first and then erase ECU failure internal memory

If flash code cannot be eliminated, notice technicians as soon as possible

Notice:

After failure elimination, erase the flash code with equipment or manually.