## Weichai power engine Euro III flash code list (for WP high pressure common rail series diesel engine)

| Flash code |  |  | Error type | Relatedparts | Problems caused | Error reason | Remedies | Flash code Lamp on |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | Error relevant to starting | ECU internal ADconverter error | ECU does not treat signal correctly; engine cannot run normally, and even cannot start | - | Cut off power and restart ECU; if there’re still problems, ECU should be replaced | √ |
| 1 | 1 | 1 |  | Synchronization signal error | Synchronization signal is the basis of fuel injection timing. Synchronization error causes abnormal fuel injection, even leading to difficult startup, giving out black smoke or engine abnormal sound, etc. | Signal tested in the crankshaft rotating speed sensor or camshaft rotation sensor is wrong | Check the connection of the two rotating speed sensors, the matching relation of high pressure fuel pump and crankshaft gear, and existence of gap between crankshaft rotating speed sensor and flywheel (whether it is between 1.0±0.5mm). | √ |
| 1 | 1 | 3 |  |  |  |  |  |  |
| 1 | 1 | 4 |  |  |  |  |  |  |
| 1 | 1 | 5 |  | ECU internal time treatment unit error | Stop the engine, and then it cannot start | - | Cut off power and restart ECU; if there’re still problems, ECU should be replaced | √ |
| 1 | 1 | 6 |  | - | Engine power is not enough | - | Cut off power and restart ECU; if there’re still problems, ECU should be replaced | √ |
| 1 | 2 | 1 |  | Starter relay error | Engine cannot start smoothly | Harness is damaged, start relay is damaged | Check start relay and wires | √ |
| 1 | 2 | 1 |  |  |  |  |  |  |
| 1 | 2 | 2 |  | T15switch error | - | - | Check T15 switch | √ |
| 1 | 2 | 3 |  | T50switch error |  | Switch is closed continually for over 120s | Check T50 switch | √ |
| 1 | 2 | 4 |  | 电池 | It is not allowable to make compression test | Battery voltage is too high (>36V）or too low(<6V) | Check battery, engine and vehicle harness |  |
| 1 | 2 | 5 |  | FMTC pulse frequency spectrum diagram | - | Calibration error | Change calibration |  |
| 1 | 3 | 1 |  | ECUinternal Sensor power module | Engine power is insufficient with black smoke | ECU internal error or air intake pressure (2.33), oil pressure sensor power (2.32) and vehicle power (24V) short-circuited or earthing | Check air intake pressure, oil pressure sensor harness (The maximum power current of SSP1 is 90mA) | √ |
| 1 | 3 | 1 |  |  | - | - | - | - |
| 1 | 3 | 1 |  |  | Engine power is not enough | ECU internal error or accelerator pedal sensor 2 power (1.84), fan rotating speed sensor power (1.68) and vehicle power short-circuited or earthing | Check accelerating pedal sensor 2, fan rotating speed sensor (The maximum power supply of SSP2 is 90mA) | √ |
| 1 | 3 | 1 |  |  | Engine power is not enough | ECU internal error or accelerating pedal sensor 1 power (1.84) rail pressure sensor power (2.13) and vehicle power short-circuited or ground | Check accelerating pedal sensor 1, rail pressure sensor (The maximum power supply of SSP3 is 50mA) | √ |
| 1 | 3 | 2 |  | Main relay error | Common rail pressure release valve (PRV) opens and engine power is insufficient | Flow calculation unit is grounded or short-circuited (3.09 3.10) | Check flow calculation unit harness | √ |
| 1 | 3 | 2 |  |  |  |  |  |  |
| 1 | 3 | 2 |  |  |  | ECU power output is grounded or short-circuited (1.04 2.03) | Check vehicle harness 2.03, 1.04 (please note: It should adopt ECU power output, and should not be connected to outer power) | √ |
| 1 | 3 | 3 | Common rail system relevant component | Rail pressure sensor | Common rail pipe release valve opens, return oil temperature rise, engine power is insufficient | Rail pressure sensor is damaged or connecting harness is short -circuited, open-circuited | Check rail pressure sensor and connecting harness, and change them when necessary | √ |
| 1 | 3 | 3 |  |  |  |  |  |  |
| 1 | 3 | 4 |  | Common rail release valve malfunctions |  | Fuel pipe problem might result from rail pressure sensor or flow calculation unit, or excessive ail pressure adjustment due to unsmooth fuel flow. | Check the fuel pipe, the rail pressure sensor and fuel flow calculation unit. | √ |
| 1 | 3 | 5 |  | Fuel flow calculation unit error |  | Fuel flow calculation unit is damaged or connecting harness is short-circuited and open-circuited. | Check fuel flow calculation unit and its connecting harness | √ |
| 1 | 3 | 5 |  |  |  |  |  |  |
| 1 | 3 | 5 |  |  |  |  |  |  |
| 1 | 3 | 5 |  |  |  |  |  |  |
| 1 | 3 | 6 | Common rail parts | Common rail pipe pressure release valve | Release vale is worn out, and not well closed, causing rail pressure to wave abnormally | Low pressure oil pipe (including fuel inlet and return) is not smooth, causing release valve to constantly open or keep too long open time. | Replace the common rail pipe | √ |
| 1 | 4 | 1 | Injector | Cylinder 1injector er | Engine power is not enough; it runs unstably and even shuts donw. | Injector harness is not well connected, worn-out harness is open-circuited, or cylinder, ect is open-circuited. | Check cylinder 1 injector | √ |
| 1 | 4 | 1 |  |  |  |  |  |  |
| 1 | 4 | 2 |  | Cylinder 2injector error |  |  | Check cylinder 2 injector |  |
| 1 | 4 | 2 |  |  |  |  |  |  |
| 1 | 4 | 3 |  | Cylinder 3injector error |  |  | Check cylinder 3 injector |  |
| 1 | 4 | 3 |  |  |  |  |  |  |
| 1 | 4 | 4 |  | Cylinder 4injector error |  |  | Check cylinder 4 injector |  |
| 1 | 4 | 4 |  |  |  |  |  |  |
| 1 | 4 | 5 |  | Cylinder 5injector error |  |  | Check cylinder 5 injector |  |
| 1 | 4 | 5 |  |  |  |  |  |  |
| 1 | 4 | 6 |  | Cylinder 6injector error |  |  | Check cylinder 6 injector |  |
| 1 | 4 | 6 |  |  |  |  |  |  |
| 1 | 5 | 1 | Control capacitor of injector | Capacitor 1(control injectors)error |  | Injector harness | Check cylinder No 1, 2 and 3 injectors harness | √ |
| 1 | 5 | 1 |  |  |  |  |  |  |
| 1 | 5 | 2 |  | Capacitor 2(control No.4.5 and 6injectors)error |  | Injector harness | Check cylinder No 4, 5 and 6 injectors harness | √ |
| 1 | 5 | 2 |  |  |  |  |  |  |
| 1 | 5 | 3 | Control inject chip | Control injector chip CY33X error |  | Internal error | Engine is off, cut off electricity and restart the engine | √ |
| 1 | 5 | 3 |  |  |  |  |  |  |
| 1 | 5 | 4 | Injection | Injector cannot reach minimum normal working | Engine stop | Harnesses of more than 3 cylinders are short-circuited or open-circuited. | Check injector harnesses on all cylinders |  |
| 1 | 5 | 5 |  | ECUinternal limit on fuel injection | - | - | - |  |
| 2 | 1 | 1 | key switches and sensors | Fuel contains water | Lamp is on when fuel contains water, water level alarm | Diesel coarse filter has gathered water in full | Maintenance of diesel filter | √ |
| 2 | 2 | 1 |  | Pedal sensor error | It cannot supply fuel to engine, rotating speed stays at 1000rpm | Harness connection or pedal sensor quality is not reliable. | Check harness connection, and replace pedal sensor if possible (Please notice pedal model before replacement！ Model error can lead to abnormal working) | √ |
| 2 | 2 | 1 |  | Pedal sensor2error |  |  |  |  |
| 2 | 2 | 2 |  | Clutch switch error | It is imposible to cruise. | Switch is not reliable or harness is open-circuited | Check switch and harness | √ |
| 2 | 2 | 3 |  | Brake switch error |  |  |  | √ |
| 2 | 2 | 3 |  | Single brake switch error |  |  |  | √ |
| 2 | 2 | 3 |  |  |  | Brake is not activated, so cruise is not allowed | Press the brake pedal before cruise | √ |
| 2 | 2 | 3 |  |  |  |  |  | √ |
| 2 | 2 | 5 |  | Trusted tests of accelerator and brake are wrong | - | Depress both the accelerator pedal and the brake pedal at the same time | - |  |
| 2 | 2 | 6 |  | Engine protection | - | Engine is over speed （>2650rpm） | - |  |
| 2 | 2 | 7 |  |  | - Torque is not enough | Transmission input torque is too much | - |  |
| 2 | 2 | 8 |  | Engine torque is over adjusted |  |  |  |  |
| 2 | 2 | 9 |  | Remote accelerator pedal | Remote accelerator pedal malfunctions | Accelerator pedal sensor is in error or harness is short-circuited or open-circuited | Check sensor harness, and replace accelerator pedal when necessary |  |
| 2 | 2 | 9 |  |  |  |  |  |  |
| 2 | 3 | 1 |  | Air intake pressure sensor | Limp Home | Sensor error or harness is short-circuited or open-circuited | Check air intake sensor harness, and replace sensor when necessary | √ |
| 2 | 3 | 2 |  | Air pressure sensor |  |  | The sensor is integrated on ECU; check that it is blocked by foreign matter. | √ |
| 2 | 3 | 3 |  | Air intake temperature sensor |  |  | Check air intake sensor harness, and replace sensor when necessary | √ |
| 2 | 4 | 1 |  | Coolant temperature sensor | Limp home, causing that intake heating cannot work normally. |  | Check water sensor harness, and replace sensor when necessary | √ |
| 2 | 4 | 2 |  | Too high water temperature alarm | Water temperature is over 105ºC, engine begins to limit running of power | Water temperature is too high | Check engine cooling system | √ |
| 2 | 4 | 3 |  | Oil pressure sensor pressure too high alarm | If instrument adopt CAN Bus, oil pressure is in error. | Oil pressure sensor is in error or oil pressure is too high | Check sensor harness, and replace sensor when necessary; check engine lubrication system | √ |
| 2 | 4 | 3 |  | Too low oil pressure alarming | - | Oil pressure is too low |  |  |
| 2 | 4 | 4 |  | Oil temperature sensor | - | Oil temperature sensor is in error or oil temperature is too low |  | √ |
| 2 | 4 | 4 |  | Too high oil temperature alarm | - | Oil temperature is too high |  |  |
| 2 | 4 | 5 |  | Water sensor test | - | - | - | - |
| 2 | 4 | 5 |  |  | - | - | - | - |
| 2 | 5 | 1 | Rail pressure monitoring | High pressure fuel pipelow pressure fuel pipe malfunctions with leakage | Starting is difficult ; Limp Home; Relief valve is open, there is plenty of high pressure fuel leakage, and fuel return temperature rises. | Inlet fuel resistance is too big; or there is inlet fuel pipe leakage; Fuel return resistance is too big; | Check fuel pipe of engine, and replace filter, fuel pipe, etc. when necessary | √ |
| 2 | 5 | 2 |  |  |  |  |  |  |
| 2 | 5 | 3 |  |  |  |  |  |  |
| 2 | 5 | 3 |  |  |  |  |  |  |
| 2 | 5 | 4 |  |  |  |  |  |  |
| 2 | 5 | 4 |  |  |  |  |  |  |
| 2 | 5 | 5 |  |  |  |  |  |  |
| 2 | 5 | 6 |  |  |  |  |  |  |
| 2 | 5 | 7 |  |  |  |  |  |  |
| 2 | 5 | 8 |  |  |  |  |  |  |
| 2 | 6 | 1 | ECU | ECU system restores | Relief valve open, Limp Home | ECU internal error | Cut off power, and restart the engine; if problem still exists, ECU should be replaced. |  |
| 2 | 6 | 1 |  |  |  |  |  |  |
| 2 | 6 | 1 |  |  |  |  |  | √ |
| 2 | 6 | 2 |  | Over speed test | - | - | - | √ |
| 2 | 6 | 2 |  |  | - | - | - |  |
| 2 | 6 | 3 | ECU internal hardware test | Chip CJ940 (broken down by external high voltage, and special attention should be paid to the protection of ECU! E.g. when welding) | Engine shuts down | Communication error | Replace ECU | √ |
| 2 | 6 | 3 |  |  |  | Internal voltage is too high |  |  |
| 2 | 6 | 3 |  |  |  | Internal voltage is too low |  |  |
| 2 | 6 | 4 |  | ECU chip and Watchdog | Engine shuts donw (cut off electricity and restart, and it can be started) | Communication error |  |  |
| 2 | 6 | 5 |  | EEPROM (Electrically Erasable Programmable Read-Only Memory） | Mileage information, etc cannot be recorded | - | Rewrite ECU (It guarantees 20 seconds of AfterRun time) |  |
| 3 | 1 | 1 | ECU vehicle related function and switch | Exhaust brake solenoid valve | Exhaust brake in failure | Exhaust brake solenoid valve (2.06) and electricity short-circuited | Check exhaust brake solenoid valve harness | √ |
| 3 | 1 | 1 |  |  |  | Exhaust brake solenoid valve (2.06) is short-circuited to ground |  |  |
| 3 | 1 | 1 |  |  |  | Exhaust brake solenoid valve (2.06) is not connected |  |  |
| 3 | 1 | 2 |  | Fan solenoid valve （PWM） |  |  | Check fan solenoid valve connection wire | √ |
| 3 | 1 | 2 |  | Fan relay |  |  |  | √ |
| 3 | 1 | 2 |  | Fan rotating speed sensor |  |  | Check fan rotating speed sensor wire |  |
| 3 | 1 | 3 |  | AC compressor relay | AC compressor does not run | AC compressor relay wire error | Check AC relay and wiring |  |
| 3 | 2 | 1 |  | Air intake heating relay | Air intake cannot work normally | Relay is short-circuited to ground or power supply | Check air intake heating relay wire |  |
| 3 | 2 | 2 |  |  | - - | - |  |  |
| 3 | 2 | 3 |  |  |  |  |  |  |
| 3 | 2 | 3 |  |  |  |  |  |  |
| 3 | 2 | 4 |  | Vehicle speed sensor | Cruise cannot be realized, shutdown switch under vehicle is in failure | Exceed maximum speed or vehicle speed signal is not reliable, voltage is unstable. | Check vehicle sensor wire, and replace the sensor if necessary | √ |
| 3 | 2 | 4 |  |  |  |  |  |  |
| 3 | 2 | 4 |  |  |  |  |  |  |
| 3 | 2 | 7 |  | Multimode switch | The functions including torque limit, rotating speed limit etc, cannot be realized | Harness or resistor error | Check mutimode switch |  |
| 3 | 3 | 1 |  | System lamp | Relevant lamp indication function is not realized | Wire is connected wrong or not connected | Check that all wires of lamps or lamps are damaged |  |
| 3 | 3 | 2 |  | Cold start lamp |  |  |  |  |
| 3 | 3 | 2 |  | Warning lamp |  |  |  |  |
| 3 | 3 | 4 |  | Water-contained fuel lamp |  |  |  |  |
| 3 | 3 | 4 |  | Warning lamp | - | - | - |  |
| 3 | 4 | 1 |  | Cruise switch | Cannot realize cruise/PTO, or cruise/PTO exits | Cruise switch has combination error | It’s recommended to use auto reset switch | √ |
| 3 | 4 | 2 |  | Exhaust brake switch | - | - | - | - |
| 3 | 4 | 3 |  | Starting switch under vehicle | It cannot start with the starting switch under vehicle | Switch gets stuck | Check switch and connection wire |  |
| 5 | 1 | 1 | External test | Engine misfire test | Engine power is insufficient, and running is unstable. | Cylinder 1 misfire | Check relevant injector, cylinder head sealing, valve gap, etc. |  |
| 5 | 1 | 2 |  |  |  | Cylinder 2 misfire |  |  |
| 5 | 1 | 3 |  |  |  | Cylinder 3 misfire |  |  |
| 5 | 1 | 4 |  |  |  | Multi-cylinders misfire |  |  |
| 5 | 3 | 1 |  |  |  | Cylinder 4 misfire |  |  |
| 5 | 3 | 2 |  |  |  | Cylinder 5 misfire |  |  |
| 5 | 3 | 3 |  |  |  | Cylinder 6 misfire |  |  |
| 5 | 3 | 4 |  | External test | - | - | - |  |
| 5 | 2 | 1 |  |  | These failures are functional indication. When activating ECU internal testing function by diagnostic instrument, if testing is successful, store flash code. |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |  |
| 5 | 2 | 3 |  |  |  |  |  |  |
| 5 | 4 | 1 | Start relevant | Synchronization signal problem | - | Flywheel processing is unacceptable | Replace flywheel | √ |