Case 1

Diesel engine series: WP10.375

Failure time: 2008.03.06

Failure description

Engine is underpowered

Failure symptom and cause analysis:

No failure code, cabin instrument indication is normal, which can exclude electrical reason. Vehicle had received mandatory maintenance 3days ago, and it was started properly, and both fuel pipe and fuel grade are normal. Test the intake pressure on starting status at original place by diagnostic instrument, the pressure is about1.012bar on idle status. When the pedal is depressed to the bottom, intake pressure has no obvious change. It is about 1.1bar, and there is doubt that air cleaner is blocked or intake system has air leakage. Dismantle air cleaner and test intake pressure, the symptom is the same. Check supercharger and intercooler, and find that there’s opening about 15cm on the lower end of intercooler (see the figure) which is retained by rubber ring and not easy to find. Dismantle aluminum welding of inter-cooler, and then after testing by user, vehicle power returns to normal.

Case 2

Diesel engine series: WP10.336

Failure time: 2008.03.16

Failure description

Engine power is not enough and it feels like no fuel supply.

Failure symptom and cause analysis

After running for a while, the engine cannot supply fuel, but there is no limit on rotating speed which can rise slowly. When in failure, faulty lamp is on with 7.a error displaying, but when the engine starts again after it shuts down, failure disappears.

After checking error code, it is found that the 7.a error is: ECU internal application is wrong or injector harness is open-circuite and is not connected, and injector has no time for electricity charging. Then replace control injector harness, failure does not disappear. After replacement of ECU, failure still exists. Finally suspect on harnesses of complete vehicle; after connecting and disconnecting conversion connectors of harness on fuse box of vehicle, failure disappears.

Annex: Poor contact of conversion connectors on vehicle harness causes abnormal electricity charging on injector harness.The condition of vehicle harness is a critical factor of source of Euro III failure.

Case 3:

Diesel series: WP12.480

Failure time: 2008.03.22

Failure description

Rotating speed losing, no power with black smoke during top gear.

Failure symptom and cause analysis:

Monitor engine intake pressure, rotating speed, injection amount. When speed increases during heavy loading, air pressure reaches 1.5 or 1.6 (absolute pressure), and pressure instantly decreases to about minus 600, injection amount returns to zero and rotating speed decreases with black smoke, but the engine does not display flash code.

Initial suspect on leakage of low pressure fuel pipeline or high pressure fuel pipeline or flattening on fuel pipe, and then check and find out that there is no problem about fuel pipe. After replacement of coarse filter, failure still exists. Check air pipeline, and no faults are found; during test on vehicle, it is found that a rubber hose between air and air cleaner is sucked to be flat when speed increase on top gear, which causes insufficient intake of engine and incomplete combustion. Engine is powerless, and speed rotation decreases. It becomes normal after replacement.

Case 4:

Failure description

1. When rotating speed reaches 1500 r during driving, rotating speed is unstable, the vehicle begins shaking. Depress on accelerator pedal, rotating speed does not increase, it shows insufficient power, and flash code lamp is on.

2. Flash code is 114 - Crankshaft rotating speed sensor signal is abnormal.

3. Test by diagnostic instrument, and find out when stepping accelerating pedal at original place, and rotating speed is over 1500 r, synchronizing signal begins jumping, which cannot be stable at 48, after checking, it finds out that crankshaft rotating speed sensor harness is normal, so it is ensured that crankshaft rotating speed sensor is in failure.

Failure symptom and cause analysis:

When diesel engine is running at high speed, crankshaft rotating speed sensor signal is unstable.

Replace crankshaft rotating speed sensor

Case 5:

Failure description

1. Diesel engine has insufficient power and there is no flash code.

2. Check multi-power fuel saving switch, and find out that there’s no change on power when gear is shifted, it is sure that switch is in failure.

Failure symptom and cause analysis:

Multi-power fuel saving switch is in failure.

Replace multi-power fuel saving switch

Case 6:

Failure description

1. There’s insufficient power with heavy loading of vehicle, so the rotating speed rises to 1500 rpm, and retains.

2. Flash code is 253 - Low pressure fuel pipe supplies fuel unsmoothly.

3. Check fuel tank, fuel pipe, filter core, and find that there’s no water in filter core, and no blocking on ventilation valve of fuel tank,

But low pressure fuel pipe is bent.

Failure symptom and cause analysis:

When the inner diameter (inner diameter is required at ≥ Φ12mm) of fuel pipe is too small or the pipe is bent, it gives rise to unsmooth of fuel supply. Repair the bent place.

Case 7:

Failure description

1. There’s no fuel supply when diesel engine is running for a while, but no limit on rotating speed. Rotating speed can rise slowly, flash code lamp is on.

2. Flash code is 143 - Cylinder 3 injector harness is open-circuited.

3. It becomes normal after replacement of injector harness.

Failure symptom and cause analysis:

Nut connected between harness and wire pole is not tightened or single cylinder injector harness is off.

First tighten harness nut of the cylinder; if no effect, replace injector harness.