Checking steps

Step 1: for vehicles equipped with multi power fuel saving switch, check if switch gear is at correct position

Step 2: read flash code, if there is no flash code, turn to next step

Step 3: check if low pressure fuel pipe and fuel quality are normal; if normal, turn to next step.

Step 4: check if air filter element, intake air pipe, and supercharger are running normally; if normal, turn to next step

Step 5: check if valve gap is normal; if normal, turn to next step

Step 6: check if connectors of sensors, accelerator pedal, vehicle harness are connected well; if normal, turn to next step

Step 7: contact workers in the authorized service station, and test as following steps by special equipment.

Check by diagnostic instrument if flywheel signal disk and fuel pump cam signal disk are in synchronization (data flow test synchronization signal 48).  If power is not enough, turn to next step

Further check by diagnostic instrument if rail pressure is built normally; if rail pressure is abnormal, maybe it is caused by injector, common rail pipe, high pressure fuel pump and ECU. Please refer to relevant information.

For vehicles equipped with multi power fuel saving switch, check that switch gear is in failure.